hostname R1

!

boot-start-marker

boot-end-marker

!

!

vrf definition Mgmt-intf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

enable secret 5 $1$4VSu$L4Q653X/.AQDcbSc6OTZN1

!

no aaa new-model

!

ip name-server 10.0.0.2

ip domain name Assurmer.local

!

!

!

!

!

!

!

!

!

!

subscriber templating

!

!

multilink bundle-name authenticated

!

!

!

!

!

license udi pid ISR4321/K9 sn FDO22350MZC

no license smart enable

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

!

!

username admin secret 5 $1$ixJw$q9TS8Dl0BjgDvA119EUU01

!

redundancy

mode none

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0/0

ip address 192.168.0.253 255.255.255.0

ip helper-address 172.16.0.1

speed 1000

no negotiation auto

!

interface GigabitEthernet0/0/0.10

encapsulation dot1Q 10

ip address 192.168.10.254 255.255.255.0

ip helper-address 172.16.0.1

!

interface GigabitEthernet0/0/0.20

encapsulation dot1Q 20

ip address 192.168.20.254 255.255.255.0

ip helper-address 172.16.0.1

!

interface GigabitEthernet0/0/0.30

encapsulation dot1Q 30

ip address 192.168.30.254 255.255.255.0

ip helper-address 172.16.0.1

!

interface GigabitEthernet0/0/0.40

encapsulation dot1Q 40

ip address 192.168.40.254 255.255.255.0

ip helper-address 172.16.0.1

!

interface GigabitEthernet0/0/0.50

encapsulation dot1Q 50

ip address 192.168.50.254 255.255.255.0

ip helper-address 172.16.0.1

!

interface GigabitEthernet0/0/0.60

encapsulation dot1Q 60

ip address 192.168.60.254 255.255.255.0

ip helper-address 172.16.0.1

!

interface GigabitEthernet0/0/0.70

encapsulation dot1Q 70

ip address 192.168.70.254 255.255.255.0

ip helper-address 172.16.0.1

!

interface GigabitEthernet0/0/0.80

encapsulation dot1Q 80

ip address 192.168.80.254 255.255.255.0

ip helper-address 172.16.0.1

!

interface GigabitEthernet0/0/0.90

encapsulation dot1Q 90

ip address 192.168.90.254 255.255.255.0

ip helper-address 172.16.0.1

!

interface GigabitEthernet0/0/0.100

encapsulation dot1Q 100

ip address 172.16.0.254 255.255.255.0

ip helper-address 172.16.0.1

!

interface GigabitEthernet0/0/0.110

encapsulation dot1Q 110

ip address 172.16.34.254 255.255.255.0

!

interface GigabitEthernet0/0/1

ip address 10.0.0.1 255.255.255.252

speed 1000

no negotiation auto

!

interface Serial0/1/0

no ip address

!

interface Serial0/1/1

no ip address

!

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

!

router rip

version 2

network 10.0.0.0

network 172.16.0.0

network 192.168.0.0

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip http client source-interface GigabitEthernet0/0/1

ip route 0.0.0.0 0.0.0.0 10.0.0.2

!

ip ssh version 2

!

!

!

!

!

control-plane

!

!

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login local

transport input ssh

!

wsma agent exec

!

wsma agent config

!

wsma agent filesys

!

wsma agent notify

!

!

end